Please note: these units are built on demand and by hand. Actual physical layout may vary from unit to unit.

# Front panel

The front panel contains either:

- On/off switch

- Red power indicator

- Green stereo indicator

- Mono/stereo switch.

Or:

- On/off switch

- Red power indicator

- Green stereo indicator

- Mono/stereo blend pot 100K: CCW=mono, CW=maximum stereo.

Or:

- Red power indicator

- Green stereo indicator

- Combined on/off, mono, stereo blend (3 positions) and stereo selector knob. Positions, reading from counter-clockwise to clockwise:

- Off

- Mono

- Stereo blend 1 (most blended: -6dB separation)

- Stereo blend 2 (middle blend: -12dB separation)

- Stereo blend 3 (least blended: -30dB separation)

- Stereo (unblended: maximum separation, typically < -40dB).

The blend positions are used in difficult reception positions.

# Rear panel

The rear panel contains:

- Power inlet, from 18VDC power pack.

- RCA input socket, usually yellow, but always on its own, spaced well away from the other two RCA sockets, for clarity. This socket receives the mono output from the FM tuner.

- Two RCA sockets together, usually black and red, for stereo output to your preamplifier: red is always the Right channel; the other socket, e.g. black, is Left.