[image: image1.jpg]“VARISLOPE STEREO” PRE-AMPLIFIER

by ILIE A K ... the first name in High Fidelity

Styled by Lonsdale-Hands for

enbanced appearance in your home.

Designed by Harold J. Leak to give
you OPTIMUM MUSICAL RESULTS

[from :—

Records . . . . . Stereo and Monaural
Tape ... . ... Stereo and Monaural
Radio . . . . . . Stereo and Monaural

Microphone . . . Stereo and Monaural




[image: image2.jpg]This pre-amplifier is the stereo version of the Varislope III monaural pre-amplifier which gained acceptance
by professional communications engineers* and music-lovers throughout the world : it incorporates facilities
which make it the most comprehensive pre-amplifier presently available.

FUNCTION

When used with any combination of two Leak monaural power amplifiers or with a ‘Stereo 20’ or
“ Stereo 50" power amplifier this pre-amplifier reproduces stereophonically and monaurally from records, tape,
radio and microphone, using any pickup, any tape head, any tuner and any microphone available in the world.
This comprehensiveness gives two important advantages. First, you can use the finest pickups and microphones
available in the world : this is not the case with cheaper low-gain pre-amplifiers. ~Second, when reproducing
from tape you can do so directly from the tape head, thus saving the cost of amplifiers built into the tape
mechanism. Note that the high sensitivity allows for the playback of the new quarter track tapes.

APPEARANCE

The * Varislope Stereo’ is styled by the world-famous designers, Richard Lonsdale-Hands Associates. The
appearance is of restrained richness and dignity—* jewel-like ™ is the description by one user—whether the unit
is used free-standing or whether it is cabinet mounted. The front panel will blend with any wood. These
were major styling necessities.

The transparent front panel is engraved on the rear face in deep maroon, cream and gold, and the case is
finished in light bronze. In appearance it matches the Leak * Trough-Line 11" F.M. Tuner.

EASE OF OPERATION

A skilled electronic engineer has no advantage whatsoever over the layman in getting the best from the
‘ Varislope Stereo’. Though comprehensive and flexible, it was created by the world’s best-known amplifier
designer specifically for the use and enjoyment of the music-lover at home, for his wife and for his children.
Simple-to-follow operating instructions are enclosed with the pre-amplifier, and five minutes practice will give
you easy mastery.

CIRCUITRY

The two identical circuits are designated “ L ™ (for the left-hand input signal) and *“ R (for the right-hand
input signal). Each circuit consists of a low-noise, low-distortion 2-stage feedback tone control pre-amplifier.
The first stage is used to give record and tape compensation by the use of frequency selective negative feedback.
The second stage embodies feedback tone control circuits which give continuously variable control of both bass
and treble frequencies, and additionally, the famous LEAK * Varislope  filter circuitry.

All controls are of the single knob, dual-ganged, close-tolerance type, which give much greater ease of
handling than the cheaper twin knob, dual-concentric, unganged type.

The pre-amplifier automatically draws its heater and anode supplies when plugged into a Leak power
amplifier.

THE “SOUTHDOWN" CABINET

The *Southdown ' cabinet, designed by Messrs. Dennis
& Robinson Ltd., provides an attractive housing for your
high fidelity equipment. It is easy to fit Leak units as
the correct cut-outs are provided.

The cabinet is finished to an unusually high standard in
either walnut or contemporary mahogany and it stands
on slender ebonised legs terminating in adjustable tilt

glides.
Price £21
* Leak amplifiers are the choice of professional engineers such as the B.B.C. (over 500 delivered), the South African Broadeasting Corporation (600),
TV and many other Commonwealth and overscas broadcasting and TV systems. who use them for transmitting and/or monitoring (quality checking}

the broadcasts to which you listen. Also. many of the gramophone records you buy are cut via LEAK amplifiers
”





[image: image3.jpg]SPECIFICATION, FACILITIES and PERFORMANCE

The sensitivities shown below give full output from Leak power amplifiers, ‘ Stereo 20 or * Stereo 507,
with a signal of 1,000 c/s, with tone controls at “ FLAT ™ and the volume control at maximum.

VALVES (Tubes) :

4—EF86 or Z729 or 6267 (International standard types
of world-wide availability).

PICKUP SELECTOR
A 2-way switch on the front panel allows the choice of :-
“ Pickup 17
Sensitivity 3-5mV at 50k ohm or 35mV at 100k ohm
selected by use of the switched input attenuator which
is fitted on the rear panel.
“ Pickup 2"
Sensitivity 3:SmV at 50k ohm or 350mV at 100k ohm
selected by use of the switched input attenuator which
is fitted on the rear panel.
This selector when used in conjunction with the Function
Switch permits the use of one of the following com-
binations :-
(1) Any two stereo pickups
(2) Any stereo pickup and any two monaural pickups
(3)  Any four monaural pickups.

INPUT SELECTOR
An eight position switch allows the choice of inputs
from :-
PICKUP. monaural and stereo
There are four playback characteristics. covering all re-
cords ever made :-
(a) RIAA. The world standard for LP records since
early 1955,
(b) NARTB. For pre-1955 LP’s and U.S.A. 78’s.
(¢}  780E (Old European). For pre-1955 78's.
(d) 78NE (New European). Since 1955, world stan-

dard for Coarse-groove (78 records). This position
also suits U.S.A. Columbia LP’s prior to 1955.

TUNER. monaural and stereo :

Sensitivity, 50mV at 100k ohm or 250 mV at 600k ohm
selected by use of the switched input attenuator which
is fitted on the rear panel.

Characteristic, flat.

TAPE HEAD, monaural and stereo :
Sensitivity, 3mV r.m.s.
Characteristic, CCIR with tone controls at * FLAT ™;
NARTB with treble control at **—2 . With moderately
priced heads, properly aligned, the playback response
will be level within plus or minus 1db from 50-10.000 ¢/s
when using EMI 74 ins./sec. test tape SRT.13. This can
usually be extended from 40-12,000 c/s by slight mani-
pulation of the tone controls. The best heads, accurately
aligned, will extend the response to 15,000 c/s.
Input impedance, 120,000 ohms approx.

MICROPHONES, monaural and stereo :
Sensitivity, 2mV r.m.s.
Characteristic, flat.
Input impedance, 120.000 ohms approx.

EXTRA. monaural and stereo :

Sensitivity, 50mV at 100k ohm or 1V at 2M ohm selected
by use of the switched input attenuator which is fitted
on the rear panel.

For (1) Any radio tuner,

(2) TV sound, or

(3) The output from equalised tape reproducers or
the like.

Characteristic, flat.

FUNCTION CONTROL
A five position switch allows the choice of :-
(1) Stereo.
(2) Stereo reverse (i.e. one can change the left-hand side
of the orchestra over to the right, and vice versa).
(3) Monaural PU (this converts a stereo pickup so that
it may play monaural LP records).

(4) R (monaural, connects both channels to the “Right”
input device).

(5) L (monaural, connects both channels to the “Left”
input device).

BASS CONTROL

Single knob, dual-ganged, close-tolerance, operative on
both channels simultaneously. Continuously variable,
plus or minus 14 db at 50 c/s.

TREBLE CONTROL

Single knob, dual-ganged, close-tolerance. operative on
both channels simultancously. Continuously variable,
plus or minus 14 db at 20,000 c/s.

FILTER and SLOPE CONTROLS

The * FILTER ” knob can be used to give very com-
prehensive control of the treble frequencies. When the
control is turned to *“ 9 ™ a filter is switched into circuit,
the turnover frequency being 9 ke/s (i.e. the frequency
at which the response falls 3 db). Other turnover fre-
quencies of 6 ke/s and 4 ke/s are also obtainable. The
*SLOPE ™ control varies the rate of attenuation above
the turnover frequency between 5 db per octave (fully
clockwise) to 25 db per octave (fully anti-clockwise).
The “ TREBLE " control is also operative at the same
time, and it can be seen that all these three controls give
increasing high-frequency attenuation as they are turned
anti-clockwise. These controls are very useful when
reproducing music in which there is high distortion at
high frequencies, making it possible to remove much of
the offensiveness whilst losing a minimum of the musical
content.

The * SLOPE ™ control is inoperative when the “FIL-
TER " is at * OFF ™.

VOLUME CONTROL WITH ON/OFF SWITCH

Single knob, dual-ganged, close-tolerance, operative on
both channels simultaneously. The switch controls the
power supply to the remotely placed Leak power
amplifier.

BALANCE CONTROL

Single knob, dual-ganged, close-tolerance. Any degree
of balance may be obtained, as either channel can be
faded to zero output without affecting the other.

RUMBLE FILTER SWITCH
Turnover, 70 c¢/s; for use with the cheaper type of
turntable.

PANEL LIGHT

This is fitted as a visual reminder that the power supply
is on when the lamp is alight.






[image: image4.jpg]FACILITIES ON THE REAR PANEL

(1) Outlets for Tape Recording :  Situated on the rear,
allowing for stereo or monaural recording from
records, tape, radio, microphone and whatever
device is connected to the “EXTRA™ input.

(2) Standard international type co-axial plugs and
sockets are provided for all input connections.

(3) Switches are provided for the adjustment of the
level of the input signals from *“PICKUP 17,
“PICKUP 27, " TUNER ", and “EXIKA".

(4)  An octal male socket. marked * FROM AMPLI-
FIER ™ accepts the female plug termination of the
4 ft. cable which is supplied for connecting the
pre-amplifier to the Leak power amplifier.

(5) A 2-pin socket marked *SWITCH ™ accepts the
plug and cable which connects to the Leak power
amplifier to control its A.C. supply.

(6) A special U-shaped bracket and wing-nut are
provided for rapid and easy fixing of the * Varislope
Stereo * in a cabinet.

TOTAL HARMONIC DISTORTION

Less than 0:01%, for 125mV output (which will load any
Leak power amplifier).

Less than 0-1% for 1:25V output (ten times the require-
ment for feeding a Leak power amplifier).

“STEREO 20"
POWER AMPLIFIER

HUM AND NOISE
When plugged into a Leak power amplifier approx-
imately 60 db below full power output on " TUNER
and “"EXTRA ™ positions, and approximately 52 do
below on other inputs, with tone controls at * FLAT ™
and volume at * MAX ™.

CROSS TALK
Between L & R channels, approx. — 50 db up to 1.000
¢/s and approx. —30 db at 10,000 ¢/s.

DIMENSIONS
Front panel, 114" x 47" (29-2 % 11-2 cms.).
Chassis, 1047 x 33" % 61" deep (26:6 X 9:5x 16°5 cms.).
This unit may be used free-standing (as on a table or
book-case), or it may be cabinet mounted on a panel of
any thickness through a cut-out of 10§” x 33" (27 x 985
cms.).

WEIGHT : 6 Ibs. (27 kgs.).
PRICE: £25-0°-0

INSTALLATION, OPERATION and MAINTENANCE
Each amplifier is accompanied by detailed instructions
and circuit drawings. Installation and operation ars
simple, as explained in the text.

Price 29 gns.

We shall be pleased to send you details of the “Sterco 20" and other LEAK products

H. J. LEAK & CO. LTD.

(ESTABLISHED 1034)

BRUNEL ROAD WESTWAY

Telephone :
SHEpherds Bush 1173 (4 lines)

PRINTED IN ENGLAND BY WILLIAMS.COOK LTD.. LONDON, ENGLAND

FACTORY

ESTATE LONDON, W3
Telegrams :
Inland: Sinusoidal, Ealux, London
Foreign: Sinusoidal, London
Ret, 110




